The current Realtek WiFi USB Linux driver had supported lots more than 10 vip/pid USB WiFi devices. However, some customers want to add new vid/pid(s) into the Realtek WiFi USB Linux driver by themselves. This document will describe the way to let the driver to support the devices with new vid/pid.

The line 62 of os_dep\linux\usb_intf.c file describes all the vid/pids which the Realtek WiFi USB Linux driver supports currently. For the new vid/pid(s) which want to be supported, they can be added into the USB id table named “rtw_usb_id_tbl” array variable.

Ex:

static struct usb_device_id rtw_usb_id_tbl[] ={

/*=== Realtek demoboard ===*/

{USB_DEVICE(0x0BDA, 0x8191)},//Default ID

/****** 8188CU ********/

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8176)},//8188cu 1*1 dongole

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8177)},//8188cu 1*1 dongole, (b/g mode only)

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8170)},//8188CE-VAU USB minCard

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x817A)},//8188cu Slim Solo

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x817B)},//8188cu Slim Combo

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x817D)},//8188RU High-power USB Dongle

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8754)},//8188 Combo for BC4

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x817E)},//8188CE-VAU (b/g mode only)

/****** 8192CU ********/

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8177)},//8191cu 1*2

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8178)},//8192cu 2*2

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x817C)},//8192CE-VAU USB minCard

/*=== Customer ID ===*/

/****** 8188CU ********/

{USB_DEVICE(0x07B8, 0x8189)},//Funai - Abocom

{USB_DEVICE(0x2019, 0xED17)},//PCI - Edimax

{USB_DEVICE(0x0DF6, 0x0052)}, //Sitecom - Edimax

{USB_DEVICE(0x7392, 0x7811)},//Edimax - Edimax

{USB_DEVICE(0x07B8, 0x8188)},//Abocom - Abocom

{USB_DEVICE(0x0EB0, 0x9071)},//NO Brand - Etop

{USB_DEVICE(0x06F8, 0xE033)},//Hercules - Edimax

{USB_DEVICE(0x103C, 0x1629)},//HP - Lite-On ,8188CUS Slim Combo

//{USB_DEVICE(0x0DF6, 0x0052)},//Sitecom - Edimax V18

//{USB_DEVICE(0x0EB0, 0x9071)},//NO Brand - Etop

//{USB_DEVICE(0x06F8, 0xE033)},//Hercules - Edimax  V18

{USB_DEVICE(0x2001, 0x3308)},//D-Link - Alpha

{USB_DEVICE(0x050D, 0x1102)},//Belkin - Edimax

{USB_DEVICE(0x2019, 0xAB2A)},//Planex - Abocom

{USB_DEVICE(0x20F4, 0x648B)},//TRENDnet - Cameo

{USB_DEVICE(0x3358, 0x13D3)},// -Azwave 8188CE-VAU

{USB_DEVICE(0x3359, 0x13D3)},//Russian customer -Azwave (8188CE-VAU  b/g mode only)

/****** 8192CU ********/

{USB_DEVICE(0x07b8, 0x8178)},//Funai -Abocom

{USB_DEVICE(0x2001, 0x3307)},//D-Link-Cameo

{USB_DEVICE(0x2001, 0x330A)},//D-Link-Alpha

{USB_DEVICE(0x2001, 0x3309)},//D-Link-Alpha

{USB_DEVICE(0x0586, 0x341F)},//Zyxel -Abocom

{USB_DEVICE(0x7392, 0x7822)},//Edimax -Edimax

{USB_DEVICE(0x2019, 0xAB2B)},//Planex -Abocom

{USB_DEVICE(0x07B8, 0x8178)},//Abocom -Abocom

{USB_DEVICE(0x07AA, 0x0056)},//ATKK-Gemtek

{}  <- the new vid/pid can be added here

};

After adding the new vid/pid into the rtw_usb_id_tbl table, the driver has to be re-compiled. The new driver image will be created and it can support the device with the new vid/pid.